FACULTY OF ENGINEERING AND TECHNOLOGY

DEPARTMENT OF WATER RESOURCES ENGINEERING

GROUP 14 WAR

| S/N | NAMES | REG.NO |
| --- | --- | --- |
|  | KALISA RONALD | BU/UP/2025/1540 |
|  | ACHOLA ELIZABETH | BU/UP/2025/3247 |
|  | OLARA EMMANUEL GEORGE | BU/UP/2025/2136 |
|  | MPUUGA MARK | BU/UP/2025/3268 |

SOLUTION OF x³ − 5x + 3 = 0

Secant Method, Newton–Raphson Method and Runge–Kutta Method
Including FOR and WHILE loop pseudocode and flowcharts

Task;

Get the math questions solved in secant, runge-kutta, and newton Raphson

Get solutions for ‘for’ and ‘while’ loop for each of the equations. Draw flowchart and get pseudo code

# 1. Problem statement

Solve the nonlinear equation

f(x) = x³ − 5x + 3 = 0

The equation has three real roots.

The following starting values are used: x₀ = 0, x₁ = 1 for the Secant method; x₀ = 1 for Newton–Raphson; and x₀ = 1 with h = 0.1 for the Runge–Kutta IV method after rewriting the root problem as the initial-value problem y′ = f(y), y (0) =1.

# 2. Secant Method

Formula:

xₙ₊₁ = xₙ − f(xₙ) (xₙ − xₙ₋₁) / [f(xₙ) − f(xₙ₋₁)]

Using x₀ = 0 and x₁ = 1 gives convergence to the root x ≈ 0.656620431.

Iteration table:

| N | xₙ₋₁ | xₙ | xₙ₊₁ | f(xₙ₊₁) | ¦Δx¦ |
| --- | --- | --- | --- | --- | --- |
| 1 | 0 | 1 | 0.750000000 | -0.328125000 | 0.250000000 |
| 2 | 1 | 0.750000000 | 0.627906977 | 0.108028224 | 0.122093023 |
| 3 | 0.750000000 | 0.627906977 | 0.658147479 | -0.005655480 | 0.030240502 |
| 4 | 0.627906977 | 0.658147479 | 0.656643089 | -0.000083983 | 0.001504389 |
| 5 | 0.658147479 | 0.656643089 | 0.656620413 | 0.000000068 | 0.000022677 |
| 6 | 0.656643089 | 0.656620413 | 0.656620431 | -0.000000000 | 0.000000018 |

Result: x ≈ 0.656620431

# 2.1. Secant Method — FOR loop pseudocode

START
x0 = 0, x1 = 1;

tolerance = 1.0000e-06

for n = 1: 50

f0 = x0^3 - 5*x0 + 3

f1 = x1^3 - 5*x1 + 3

x2 = x1 - f1*(x1-x0)/(f1-f0)

if abs(x2-x1) < tolerance

break

end

x0 = x1

x1 = x2

end

fprintf ('Root = %.9f\n', x2);
END

# 2.2. Secant Method — WHILE loop pseudocode

START

x0 = 0;

x1 = 1;

tolerance = 1.0000e-06;

n = 0

while n < 50

f0 = x0^3 - 5*x0 + 3

f1 = x1^3 - 5*x1 + 3

x2 = x1 - f1*(x1-x0)/(f1-f0)

n = n + 1

if abs(x2-x1) < tolerance

break

end

x0 = x1

x1 = x2

end

fprintf ('Root = %.9f\n', x2);
END

# 3. Newton–Raphson Method

f′(x) = 3x² − 5

Formula: xₙ₊₁ = xₙ − f(xₙ)/f′(xₙ)

| N | xₙ | xₙ₊₁ | f(xₙ₊₁) | ¦Δx¦ |
| --- | --- | --- | --- | --- |
| 0 | 1 | 0.500000000 | 0.625000000 | 0.500000000 |
| 1 | 0.500000000 | 0.647058824 | 0.035619784 | 0.147058824 |
| 2 | 0.647058824 | 0.656572795 | 0.000176568 | 0.009513972 |
| 3 | 0.656572795 | 0.656620430 | 0.000000004 | 0.000047634 |
| 4 | 0.656620430 | 0.656620431 | 0.000000000 | 0.000000001 |

Result: x ≈ 0.656620431

Newton’s Raphson Method

# 3.1. Newton–Raphson — FOR loop pseudocode

start

x = 1;

tolerance = 1.0000e-06

for n = 1: 50

fx = x^3 - 5*x + 3

dfx = 3*x^2 - 5

xnew = x - fx/dfx

if abs(xnew-x) < tolerance

break

end

x = xnew

end

fprintf ('Root = %.9f\n', xnew)

stop

3.2. Newton–Raphson — WHILE loop pseudocode

START
x = 1;

tolerance = 1.0000e-06;

n = 0;

while n < 50

fx = x^3 - 5*x + 3

dfx = 3*x^2 - 5

xnew = x - fx/dfx

n = n + 1

if abs(xnew-x) < tolerance

break

end

x = xnew

end

fprintf ('Root = %.9f\n', xnew);
END

# 4. Runge–Kutta 4th Order (RK4)

RK4 is an ODE solver rather than a conventional root-finding method. For the requested equation, we can use the auxiliary IVP y′ = y³ − 5y + 3, y (0) =1 and demonstrate the RK4 algorithm. This does not by itself define a root of the original equation; the equilibrium values of the IVP satisfy y³ − 5y + 3 = 0.

For y′ = F(y), the RK4 steps are:

k1 = F(yₙ)
k2 = F (yₙ + h k1/2)
k3 = F (yₙ + h k2/2)
k4 = F (yₙ + h k3)
yₙ₊₁ = yₙ + (h/6) (k1 + 2k2 + 2k3 + k4)

Use h = 0.1 and y₀ = 1. The RK4 iteration can be repeated with a FOR or WHILE loop.

3. Flow chart

START

Input y0, h, N, n = 0

Calculate k1.  k1 = f(y)

Calculate new y y = y+h/6(k1+2k2+2k3+k4)

Calculate k2.   k2 = f(y+hk1/2)

Calculate k3.  k3 = f(y+hk2/2)

Calculate k4. k4 = f(y+k3)

n = n + 1

No

Is n < N?

Yes

print y

END

# 4.1. RK4 — FOR loop pseudocode

START
y = 1, h = 0.1, N = 10

for n = 1 : N

k1 = y^3 - 5*y + 3

k2 = (y + h*k1/2) ^3 - 5*(y + h*k1/2) + 3

k3 = (y + h*k2/2) ^3 - 5*(y + h*k2/2) + 3

k4 = (y + h*k3) ^3 - 5*(y + h*k3) + 3

y = y + (h/6) *(k1 + 2*k2 + 2*k3 + k4)

fprintf('n = %d, y = %.6f\n', n, y);
END

# 4.2. RK4 — WHILE loop pseudocode

START
y = 1;

h = 0.1;

n = 0;

N = 10

while n < N

k1 = y^3 - 5*y + 3

k2 = (y + h*k1/2) ^3 - 5*(y + h*k1/2) + 3

k3 = (y + h*k2/2) ^3 - 5*(y + h*k2/2) + 3

k4 = (y + h*k3) ^3 - 5*(y + h*k3) + 3

y = y + (h/6) *(k1 + 2*k2 + 2*k3 + k4)

n = n + 1

fprintf('n = %d, y = %.6f\n', n, y);

end
END

# Summary of results

For x³ − 5x + 3 = 0, the three real roots are approximately:

-2.490864

0.656620

1.834243